Exercise 3.2: Analyzing Network Infrastructure with NET Analyzer

Wi-Fi Network Analysis (NET Analyzer) - By Location

1) Router & Gateway Details (same for all locations)

- SSID: DNA-WIFI-FD20

- BSSID (AP MAC): 58:00:32:90:fd:22

- Vendor: Genexis B.V.

- Device IP address: 192.168.1.103

- Subnet mask: 255.255.255.0

- Default gateway (router) IPv4: 192.168.1.1

- DNS server IPv4: 192.168.1.1

- IPv6: enabled (IPv6 addresses shown)

- Default gateway IPv6: fe80::5a00:32ff:fe90:fd20

- DNS server IPv6: 2001:14ba:9ca1:ff00::1

- DHCP lease time: 12h

2) Wi-Fi Radio Configuration (same for all locations)

- Band: 2.4 GHz

- Operating channel: 1 (2412 MHz)

- Wi-Fi standard: 802.11n

- Max. speed (Down/Up): 300 / 300 Mbps (link capability shown by the app)

Data Collection (by location)

Bedroom

- Speed (Down / Up): 5 / 26 Mbps

- Signal strength: -48 dBm (strong)

Kitchen

- Speed (Down / Up): 5 / 43 Mbps

- Signal strength: -68 dBm (medium)

Outside of the apartment

- Speed (Down / Up): 7 / 39 Mbps

- Signal strength: -74 dBm (weaker)

Network Speed & Latency

Your screenshots include throughput (Down/Up) but not the ping/latency results.
To complete the “latency” requirement, run NET Analyzer Ping tests and record:

- Ping to gateway 192.168.1.1 (local Wi-Fi latency)

- Ping to 1.1.1.1 or 8.8.8.8 (internet path latency)
Include average ms + packet loss.

Analysis & Troubleshooting

A) Signal strength vs. speed

- The strongest signal occurred in the bedroom (-48 dBm), but download speed was still low (5 Mbps).

- Outside had the weakest signal (-74 dBm), but download speed was similar (7 Mbps).

Interpretation: Performance is not strongly limited by Wi-Fi signal strength alone in this case. The consistently low download suggests other limiting factors, such as:

- 2.4 GHz congestion (neighbors sharing the band),

- channel contention/interference, or

- an ISP/router-side download limit at that moment.

B) Channel overlap / interference risk

- All measurements use 2.4 GHz channel 1.

- 2.4 GHz is commonly crowded; even with good signal, heavy channel sharing can reduce throughput and increase latency.

C) Suboptimal placement / coverage

- Signal drops from -48 dBm (bedroom) → -74 dBm (outside), showing clear attenuation from distance + walls.

- If outdoor coverage matters, you may need better placement or added coverage equipment.

Practical Solutions to Improve Performance

1) Reduce congestion (2.4 GHz tuning)

- Scan nearby Wi-Fi networks and pick the least congested channel among 1 / 6 / 11.

- Set 2.4 GHz channel width to 20 MHz (more stable in crowded buildings).

2) Use a different frequency band

- If your router supports it, switch to 5 GHz (802.11ac/ax) for:

  - higher speed potential,

  - less interference,

  - often better latency.

3) Router repositioning

- Place router more centrally, higher, away from:

  - thick walls, metal surfaces,

  - kitchen appliances (microwave interference is common).

4) Extend coverage if needed

- For better signal outside: add a mesh node or extender placed halfway between the router and the weak area.

5) Security/config

- Use WPA2-AES or WPA3 if available.

- Disable WPS if it’s enabled.