Exercise 3.3: Exploring Bluetooth Low Energy (BLE) Networks with BLE Scanner

Data Collection

Recorded fields (from BLE Scanner app):

- Device name (if available)

- RSSI (dBm)

- MAC address / device identifier

Classroom (from classroom.jpeg)

- 5 devices clearly recorded

- 3 connectable, 2 non-connectable

- RSSI range: -58 to -82 dBm

Library (from library.jpeg)

- 5 devices clearly recorded

- 1 connectable (Polar wearable), 4 non-connectable

- RSSI range: -64 to -79 dBm

Analysis

1) Relationship between RSSI and estimated distance

In both locations, the app’s estimated distance generally follows the expected trend:

- Stronger RSSI (less negative) → shorter estimated distance

- Weaker RSSI (more negative) → longer estimated distance

Examples:

- Classroom:

  - -58 dBm → ~0.89 m (closest/strongest)

  - -82 dBm → ~14.13 m (weakest/farthest)

- Library:

  - -64 dBm → ~1.78 m (strongest in library screenshot)

  - -78 to -79 dBm → ~8.91–10.00 m (weaker/farther)

But it’s not perfectly consistent, which is normal. RSSI-based distance is noisy because:

- Different devices use different transmission power (Tx power) and antenna designs.

- People moving, body blocking, and device orientation change signal a lot.

- Multipath reflections (signals bouncing off walls/furniture) can make RSSI jump.

So: RSSI gives a rough distance hint, not a true measurement.

2) Number and types of BLE devices in different environments

Classroom

- More connectable devices visible (3 connectable).

- Likely more personal devices nearby (phones/laptops/wearables) advertising connectable services.

Library

- Mostly non-connectable devices (4 non-connectable), plus one identifiable wearable (Polar mobile).

- Libraries often contain many passive BLE advertisers (beacons, nearby phones set to limited advertising, etc.), but fewer devices may allow connection openly.

Key difference:

- Classroom: higher proportion of connectable devices

- Library: higher proportion of non-connectable advertisers

3) Patterns, inconsistencies, anomalies

- Many devices show Name = N/A in both places, even when connectable. This is common because device names may not be included in advertising packets, or the app may not decode them.

- Estimated distance sometimes depends heavily on the device’s Tx power and environment. Two different devices at the same real distance can show very different RSSI/distance estimates.

- The Polar device in the library is clearly identifiable (a wearable/fitness ecosystem). That’s an example of a “type” of BLE device you can infer from advertising.

Environmental factors affecting results

Transmission power (Tx power)

- Devices with higher Tx power (or better antenna) appear “closer” than they really are compared to low-power beacons.

Physical obstructions

- Walls, shelves, books, and human bodies absorb 2.4 GHz signals.

- Library layouts (shelves) can strongly attenuate or reflect BLE signals.

Environmental noise/interference

- BLE operates at 2.4 GHz, the same band as Wi-Fi and many other devices.

- Congested 2.4 GHz environments can increase RSSI fluctuation and scanning inconsistency.

Security and Privacy Considerations

How BLE is used in everyday devices

- Wearables: fitness bands, heart-rate monitors (e.g., Polar)

- IoT devices: sensors, smart locks, trackers, smart home devices

- Beacons: retail/indoor navigation/asset tracking

- Accessories: earbuds, keyboards, mice

Risks and implications

- Device tracking:
Even passive scanning can reveal that a specific device is nearby repeatedly. If identifiers are stable, a person could be tracked over time.

- Identifier exposure:
BLE advertisements can expose device identifiers and sometimes manufacturer/service hints (e.g., wearables), allowing profiling.

- Passive scanning risks:
Anyone can scan without connecting, which means presence detection is easy (who is nearby / when a device appears).

Mitigations (practical)

- Many modern devices use randomized/private BLE addresses, which reduces long-term tracking (though not always perfectly).

- Users can reduce exposure by disabling Bluetooth when not needed, or disabling “discoverable” / limiting advertising where possible.

Final Conclusion

BLE scanning in two environments (classroom and library) detected multiple nearby devices, often without names but with visible identifiers and RSSI values. In both locations, RSSI loosely correlated with estimated distance (stronger signals generally appeared closer), but results were not perfectly consistent due to real-world factors like transmission power differences, multipath reflections, obstructions, and 2.4 GHz interference. The classroom environment showed a higher proportion of connectable devices, while the library scan showed mostly non-connectable advertisers plus one clearly identifiable wearable (Polar). Finally, the scan highlights real privacy concerns: even without connecting, BLE advertisements can enable passive presence detection and potential device tracking, making identifier protection and address randomization important for everyday BLE devices.